- New Hilux 2020
Dynamic Press Launch – Technical data

- Overall Vehicle Specifications & Dimensions

|  |  | 2.4 D-4D 4x2 6MT | 2.4 D-4D 4x4 6MT |  | 2.4 D-4D 4x4 6AT | 2.8 D-4D 4x4 6MT | 2.8 D-4D 4x4 6AT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ENGINE |  |  |  |  |  |  |  |
| Engine code |  | 2GD-FTV |  |  |  | 1GD-FTV |  |
| Type |  | 4-in line cylinders |  |  |  |  |  |
| Fuel type |  | Diesel |  |  |  |  |  |
| Valve mechanism |  | 16-valve DOHC |  |  |  |  |  |
| Displacement (cm³) |  | 2,393 |  |  |  | 2,755 |  |
| Bore x Stroke (mm) |  | 92.0 x 90.0 |  |  |  | 92.0 x 103.6 |  |
| Compression ratio (:1) |  | 15.6 |  |  |  |  |  |
| Max. power (DIN hp) kw @ rpm |  | (150)110 @ 3,400 |  |  |  | (204)150 @ 3,400 | (204)150 @ 3,400 |
| Max. torque (Nm @ rpm) |  | 400 @ 1,600-2,000 |  |  |  | 420 @ 1,400-3,400 | 500 @ 1,600-2,800 |
| TRANSMISSION |  |  |  |  |  |  |  |
| Type |  | 6MT |  |  | 6AT | 6MT | 6AT |
| Gear ratios | 1st | 4.784 |  |  | 3.600 | 4.784 | 3.600 |
|  | 2nd | 2.423 |  |  | 2.090 | 2.423 | 2.090 |
|  | 3rd | 1.443 |  |  | 1.488 | 1.443 | 1.488 |
|  | 4th | 1.000 |  |  | 1.000 | 1.000 | 1.000 |
|  | 5th | 0.777 |  |  | 0.687 | 0.826 | 0.687 |
|  | 6th | 0.643 |  |  | 0.580 | 0.643 | 0.580 |
|  | Reverse | 4.066 |  |  | 3.732 | 4.066 | 3.732 |
| Differential gear ratio (final drive) |  | 3.583 |  |  | 4.100 | 3.583 | 3.909 |
| BRAKES |  |  |  |  |  |  |  |
| Front |  | Ventilated disc brake with fixed 4-pod caliper |  |  |  |  |  |
| Rear |  | Leading-trailing drum brake |  |  |  |  |  |
| Additional features |  | ABS, BA, EBD, VSC, TSC, HAC |  | ABS, BA, EBD, VSC, TSC, A-TRC, DAC, HAC |  |  |  |
| SUSPENSION |  |  |  |  |  |  |  |
| Front |  | Double wishbone |  |  |  |  |  |
| Rear |  | Leaf spring rigid axle |  |  |  |  |  |
| STEERING |  |  |  |  |  |  |  |
| Steering gear type |  | Rack & pinion |  |  |  |  |  |
| Power steering type |  | Hydraulic with variable flow control valve |  |  |  |  |  |
| Min. turning circle tyre/body (m) |  | 12.8 / 13.4 |  |  |  |  |  |
| TIRES AND WHEELS |  |  |  |  |  |  |  |
| Tires |  | 225/70R17C 265/65R17 265/60R18 |  |  |  | 265/65R17 265/60R18 |  |
| Wheels |  | 17 x 6J 17 x 7 1/2J 18 x 7 1/2J |  |  |  | - 17 x 7 1/2J 18 x 7 1/2J |  |

|  | 2.4 D-4D |  |  |  |  |  |  | 2.8 D-4D |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Single Cab |  | Extra Cab |  | Double Cab |  |  | Extra Cab |  | Double Cab |  |
|  | 4x2 M/T | 4x4 M/T | 4x2 M/T | 4x4 M/T | 4x2 M/T | 4x4 M/T | 4x4 A/T | 4x4 M/T | 4x4 A/T | 4x4 M/T | 4x4 A/T |
| PERFORMANCE |  |  |  |  |  |  |  |  |  |  |  |
| Max. speed (km/h) | 175 | 170 | 175 | 170 | 175 | 170 | 170 | 175 | 175 | 175 | 175 |
| Acceleration (0-100 km/h) | 13.2 | 13.2 | 13.2 | 13.2 | 13.2 | 13.2 | 12.8 | 10.1 | 10.7 | 10.1 | 10.7 |
| FUEL CONSUMPTION – WLTP combined cycle |  |  |  |  |  |  |  |  |  |  |  |
| Combined (l/100 km) | 8.8 to 9.5 | 8.9 to 10.3 | 8.8 to 9.5 | 8.9 to 10.3 | 8.8 to 9.8 | 8.9 to 10.3 | 8.7 to 10.1 | 7.8 to 8.9 | 8.8 to 10.1 | 7.8 to 8.9 | 8.8 to 10.1 |
| Fuel tank capacity (l) | 80 |  |  |  |  |  |  |  |  |  |  |
| CO2 EMISSIONS – WLTP combined cycle |  |  |  |  |  |  |  |  |  |  |  |
| Combined (g/km) | 233 to 252 | 235 to 274 | 233 to 252 | 235 to 274 | 233 to 260 | 235 to 274 | 229 to 267 | 207 to 236 | 233 to 266 | 207 to 236 | 233 to 266 |

|  | 2.4 D-4D |  |  |  |  |  |  | 2.8 D-4D |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Single Cab |  | Extra Cab |  | Double Cab |  |  | Extra Cab |  | Double Cab |  |
|  | 4x2 M/T | 4x4 M/T | 4x2 M/T | 4x4 M/T | 4x2 M/T | 4x4 M/T | 4x4 A/T | 4x4 M/T | 4x4 A/T | 4x4 M/T | 4x4 A/T |
| WEIGHT |  |  |  |  |  |  |  |  |  |  |  |
| Curb weight min-max (kg) | 1,700-1,725 | 1,800-1,850 | 1,790-1,825 | 1,915-1,965 | 1,955-2,005 | 2,100-2,160 | 2,095-2,155 | 2,090-2,300 | 2,090-2,300 | 2,120-2,335 | 2,125-2,340 |
| Gross vehicle weight (kg) |  |  | 3,000 | 3,150 | 3,050 | 3,210 | 3,210 | 3,150 | 3,150 | 3,210 | 3,210 |
| TOWING CAPACITY |  |  |  |  |  |  |  |  |  |  |  |
| Braked (kg) | 3,000 | 3,500 | 3,000 | 3,500 | 3,000 2,800* | 3,500 | 3,500 | 3,500 | 3,500 | 3,500 | 3,500 |
| Unbraked (kg) | 750 | 750 | 750 | 750 | 750 | 750 | 750 | 750 | 750 | 750 | 750 |

*Thailand production

|  | Single Cab | Extra Cab | Double Cab |
| --- | --- | --- | --- |
| EXTERIOR DIMENSIONS |  |  |  |
| Overall length (mm) | 5,325 | 5,325 | 5,325 |
| Overall width (mm) | 1,800 | 1,855 / 1,900* | 1,855 / 1,900* |
| Overall height (mm) | 1,795 | 1,810 | 1,815 |
| Wheelbase (mm) | 3,085 | 3,085 | 3,085 |
| Tread front (mm) | 1,495 | 1,535 | 1,535 |
| Tread rear (mm) | 1,510 | 1,550 | 1,550 |
| Front overhang (mm) | 1,000 | 900 | 900 |
| Rear overhang (mm) | 1,250 | 1,250 | 1,250 |
| Min. running ground clearance (mm) | 277 | 289 | 293 |
| Angle of approach (degrees) | 30 | 31 | 31 |
| Angle of departure (degrees) | 25 | 26 | 26 |
| Drag coefficient (Cd) | 0,394** |  |  |
| INTERIOR DIMENSIONS |  |  |  |
| Interior length (mm) | 932 | 1,404 | 1,697 |
| Interior width (mm) | 1,480 | 1,480 | 1,480 (front) / 1,410 (rear) |
| Interior height (mm) | 1,175 | 1,170 | 1,170 |
| Couple distance (mm) | N/A | N/A | 841 |
| Head room (mm) | 997 | 997 (front) / 969 (rear) | 997 (front) / 969 (rear) |
| DECK |  |  |  |
| Length (mm) | 2,315 | 1,810 | 1,525 |
| Max. width (mm) | 1,645 | 1,645 | 1,645 |
| Height (mm) | 480 | 480 | 480 |

*’Invincible’ grade only		** Depending on vehicle specification