|  | New AYGO (normal version) |  | New AYGO (ECO version) |
| --- | --- | --- | --- |
| ENGINE |  |  |  |
| Type | 1.0 VVT-i |  |  |
| Fuel type | Petrol |  |  |
| Valve mechanism | 4-valve DOHC |  |  |
| Displacement (cm²) | 998 |  |  |
| Bore x stroke (mm) | 71 x 84 |  |  |
| Compression ratio ( :1) | 11.8 |  |  |
| Max. power (DIN hp) kW @ rpm | (72) 53 @ 6,000 |  |  |
| Max. torque (Nm @ rpm) | 93 @ 4,400 |  |  |
| BRAKES |  |  |  |
| Front (diameter x thickness mm) | Disc (247 x 20) |  |  |
| Rear (inner diameter mm) | Drum (200) |  |  |
| SUSPENSION |  |  |  |
| Front | MacPherson strut |  |  |
| Rear | Torsion beam |  |  |
| STEERING |  |  |  |
| Type | Rack & Pinion |  |  |
| Additional feature | Electric Power Steering (EPS) |  |  |
| Overall ratio | 14.3 (on 14 inch) / 14.4 (on 15 inch) |  |  |
| Lock to lock | 2.75 (on 14 inch) / 2.60 (on 15 inch) |  |  |
| Min. turning circle (m) | 9.6 (on 14 inch) / 10.2 (on 15 inch) |  |  |
| TRANSMISSION |  |  |  |
| Type | M/T | x-shift | M/T |
| GEAR RATIOS |  |  |  |
| 1st | 3.545 | 3.545 | 3.545 |
| 2nd | 1.913 | 1.913 | 1,913 |
| 3rd | 1.310 | 1.161 | 1.310 |
| 4th | 1.027 | 0.973 | 0.973 |
| 5th | 0.850 | 0.804 | 0.804 |
| Reverse | 3.214 | 3.214 | 3.214 |
| Differential gear ratio (:1) | 3.550 | 4.294 | 3.550 |
| PERFORMANCE |  |  |  |
| Max. speed (km/h) | 160 | 160 | 160 |
| 0-100 km/h (sec) | 13.8 | 15.2 | 13.8 |
| FUEL CONSUMPTION |  |  |  |
| Combined (l/100 km) | 4.1 | 4.2 | 3.8 |
| Fuel tank capacity (l) | 35 | 35 | 35 |
| CO2 EMISSIONS |  |  |  |
| Combined (g/km) | 93 | 95 | 86 |
| VEHICLE WEIGHT |  |  |  |
| Curb weight (kg) | 840 – 905 | 860 - 910 | 850 – 915 |
| Gross vehicle weight (kg) | 1,240 | 1,240 | 1,240 |
| EXTERIOR DIMENSIONS |  |  |  |
| Overall length (mm) | 3,465 |  |  |
| Overall width (mm) | 1,615 |  |  |
| Overall height (mm) | 1,460 |  |  |
| Front overhang (mm) | 685 |  |  |
| Rear overhang (mm) | 440 |  |  |
| Wheelbase (mm) | 2,340 |  |  |
| Tread (mm) front | 1,430 |  |  |
| Tread (mm) rear | 1,420 |  |  |
| Drag coefficient (Cd) | 0.28 |  |  |
| Wheel size (inch) | 14×4 1/2J or 15×4 1/2J |  |  |
| Tyre size | 165/65R14 79T or 165/60R15 77H |  |  |
| INTERIOR DIMENSIONS |  |  |  |
| Interior length* (mm) | 1,630 |  |  |
| Interior width (3/5-door mm) | 1,250 / 1,300 |  |  |
| Interior height (mm) | 1,205 (normal) 1,155 (canvas top) |  |  |
| Couple distance | 806 |  |  |
| Luggage space (liter) | 168 |  |  |

* from accelerator pedal to the rear seat hip point