Hatz Diesel presents a new engine family at bauma 2013

Hatz Diesel comes up with a whole host of new products at this year's bauma in Munich. In addition to the exhibition highlight, the world premiere of a water-cooled 4-cylinder engine with Bosch common rail system, the serial production Intelligent Power Pack (iPP), as well as the L-series with separable DPF, will be presented.

Ruhstorf, January 29, 2013. The German based company Motorenfabrik Hatz comes up with a variety of new products at this year's bauma in Munich. Most importantly, the world premiere of a new generation of water-cooled engines, which in particular is characterized by its compact design. In addition, a production ready model of the revised L-series, which is equipped with exhaust gas recirculation (EGR) and diesel particulate filter (DPF) in order to comply with the emission standards EU Stage IIIB and EPA Tier IV final. Furthermore, the business field 'Hatz Systems', which develops and sells entire systems based on the Hatz diesel engines, shows the series production Intelligent Power Pack (iPP).

All exhibits will be shown on a newly designed stand in Hall A4 Stand 423/528.

World premiere: new H-series engine family

From 2014 a new engine family will expand the engine line-up of Motorenfabrik Hatz, which is celebrating its world premiere at bauma 2013 with the 4-cylinder model. Sales of the 2-litre diesel engine, which is equipped with a Bosch common rail system, start at the beginning of 2014. Like all Hatz engines, this engine also brings a number of technical advancements with it, which leads to the fact that the engine is the most compact in its class, and has the best power density compared to its competitors.

With 1951 cc, the engine has a maximum power of 56 kW and a maximum torque of 240 Nm, which is already available from as low as 1600 rpm. Besides a main power take-off, an auxiliary power take-off is provided, which can be permanently loaded with up to 130 Nm. Due to an optimized combustion chamber geometry, combined with the sophisticated technology of the Bosch common rail system, which operates with injection pressures of up to 1800 bars, the particulate emissions are considerably reduced. Therefore it is possible to achieve the exhaust emissions standards EU 97/68 stage IIIB and EPA Tier IV final without the use of a diesel particulate filter.
Apart from the technical data the motor can also score points with low cost of ownership; firstly because of a low fuel consumption of 210 g / kWh (best point), and secondly because of a long service interval of 500 hours. The latter is mainly because of the 7 litre oil sump and maintenance-free hydraulic valve train adjustment.

L- and M-series with separable diesel particulate filter

Not least because of the recent changes of the exhaust emissions regulation in the European Union and the United States the proven L-and M-series engine has been revised. The 3 - and 4-cylinder versions of both engine series will be available with exhaust gas recirculation, separable diesel particulate filter and oxidation catalyst in the 2nd Quarter of 2013. Both engine series comply with the exhaust legislation in the EU and the U.S., EU97/68 Stage IIIb and EPA Tier IV final respectively.

The exhaust after treatment including the associated components such as HC-evaporator can be mounted in various positions, thus contributing to many uses.
The L-and M-series have significantly contributed to the fact, that the Hatz engines are known for their robustness and durability.

The permanent magnet generator - iPP

The iPP - intelligent power pack - which is developed and produced by the division 'Hatz Systems' is on display in its final equipment level at the Hatz bauma stand. The engine speed of the 3.5 kVA generator unit is completely variable and will adjust automatically depending on the required load. This provides a reduced usage of consumables, extended maintenance intervals and reduces wear. In addition, the generator impresses with its compact dimensions and reduced operating noise of just 66 dB (A) in a 7m radius.

With dimensions of just 536 x 630 x 510 mm (height x width x depth), corresponding to a volume of about 172 litres, and a weight of about 120 kg, the iPP is a compact and lightweight diesel generator for professional use. The basis is a Hatz 1B30V with vertical crankshaft, which is the perfect engine for this application not just because of its size.

The iPP is equipped with a permanent magnet generator (PMG) which is directly integrated into the engine flywheel. Thus the generator is saving space and is cooled efficiently in the air supply of the engine. Unlike conventional generators, a PMG is maintenance-free, with no significant wear and with a high efficiency of up to 90%.

In the standard configuration the iPP provides a voltage of 230 V at 50 Hz, on request, also a 60 Hz version is available.

1D81 hydro concept

In addition to the world premiere of the H-Series, another water-cooled model will be presented: the 1D81 hydro concept. The basis of the prototype consists of a robust and reliable 1D81, which is equipped with a new cylinder and head with water cooling.

The special feature of the new system is the compact design of the overall system. Using the existing flywheel along with a compact radiator and having not a single external waterhose, the engine is only slightly larger than its air cooled counterpart.

With the prototype the effects on engine performance, noise and exhaust emissions through the use of water cooling are examined.

<ENDS>

Pictures

H-Series test bench1.jpg	New H-series at the emissions test bench

H-Series test bench2.jpg 	New H-series at the emissions test bench

H-Series test bench3.jpg	New H-series at the emissions test bench

DPF.jpg			Separable DPF for L- and M-series engines

4L43C_EGR.jpg		4L43C plexiglas show model with EGR and DPF

iPowerPACK.jpg		Hatz iPP - PMG with inverter technology

About Motorenfabrik Hatz

Founded in 1880 by Mathias Hatz, the company has become the world leader in diesel engines up to 56 kW and in its 130-year history it is still a family managed business today. The engines are mainly used in construction equipment, compressors, commercial vehicles, agricultural machines and equipment and are popular for their high quality and robustness.

Worldwide, the Motorenfabrik Hatz employs approximately 1,000 employees in 13 Branches, 94 offices and 500 service centers in over 100 countries, thus ensuring a high quality service and efficient spare parts supply.

Besides engine manufacturing, the company produces high-precision metal components in its business field components, especially for the automotive and commercial vehicle industry, agriculture and engineering. In addition, the company has a high competence in system engineering and develops and produces special equipment based on industrial engines and offers its customers tailor-made solutions.